College of Science and Engineering

Department of Electrical and Computer Engineering

EE 2361 Final Project

Temperature-Controlled Fan

Yuan Jiang

jian0655@umn.edu

Soorya Sundravel

sundr030@umn.edu

Hongyu Wang

wang9529@umn.edu

Thomas Skrzek

skrze005@umn.edu

Introduction

Modern mechanical and electrical systems require temperature control to ensure that they are functioning properly. Proper airflow and operating temperature help the components maintain their peak working efficiency. This project aims to offer a solution by implementing a fan that turns on automatically at a certain temperature. When the fan is turned on, its speed could be changed manually by the press of a button. This involves reading the data through a temperature sensor and controlling the speed of the fan using a motor driver. The fan turns off and the button is disabled when the temperature falls below a certain threshold. An LCD display is used to report the temperature values.

The libraries used in this project involve setting up the respective devices and reading data from, or writing data to them. Their intended function is to perform the tasks as stated above. There are three libraries created for this project: Temperature Sensor, Motor Driver, and LCD Display.

The temperature sensor library initializes the sensor by establishing I2C protocol, setting baud rate, and writing commands to receive high precision temperature data. The library also implements a function to read data from the device by sending an I2C packet with the sensor slave address and read command.

The motor driver library initializes Output Compare, as PWM signals are used to control the speed of the fan. Input Capture is also set up in this library to detect button presses and toggle between speed settings of the fan. An Input Capture Interrupt is called every time the button is pressed, which in turn calls the setSpeed function to change the speed of the fan using OC1RS.

The LCD library configures the display by sending instruction bytes and provides functionality for printing characters and strings using the I2C protocol. A delay function is also implemented to be used in the initialization.

Hardware Description

The PIC24FJ64GA002 microcontroller was used in this project. The microcontroller was interfaced with the SHT40 Humidity and Temperature Sensor, TB6612FNG Motor Driver, and the AQM0802A-RN-GBW LCD Display. A 5V DC source is connected to power the Motor Driver, along with a push button wired to the microcontroller. A DC motor with attached fan blades is connected to the output pins of the Motor Driver. The hardware block diagram and connection of the system are shown below in Fig. 1.

Figure 1: Hardware Block Diagram and Connection of Temperature-Controlled Fan

Devices Used:

- PIC24FJ64GA002

  - Overview: https://www.microchip.com/en-us/product/PIC24FJ64GA002

  - Datasheet: https://ww1.microchip.com/downloads/en/devicedoc/39881e.pdf

- SHT40 Humidity and Temperature Sensor

  - Overview: https://www.adafruit.com/product/4885

  - Datasheet: https://cdn-learn.adafruit.com/assets/assets/000/099/223/original/Sensirion_Humidity_Sensors_SHT4x_Datasheet.pdf?1612388531

- TB6612FNG Motor Driver

  - Overview: https://www.sparkfun.com/products/14451

  - Datasheet: https://www.sparkfun.com/datasheets/Robotics/TB6612FNG.pdf

- AQM0802A-RN-GBW LCD Display

  - Overview: https://international.switch-science.com/catalog/1407/

  - Datasheet: https://akizukidenshi.com/download/ds/xiamen/AQM0802.pdf

- DC Motor

  - Overview: https://www.adafruit.com/product/711

Pins Used:

- Microcontroller (PIC24FJ64GA002)

  - SCLx, SDAx pins for I2C (Temp Sensor and LCD)

  - Output Compare (Motor Driver)

  - Input Capture (Push Button)

- Motor Driver (TB6612FNG)

  - PWMA -> RB6

  - AIN2 -> RB7

  - AIN1 -> RB8

  - STBY -> RB9

- Humidity & Temp Sensor (SHT40)

  - SCL -> SCL2

  - SDA -> SDA2

- LCD Display (AQM0802A-RN-GBW)

  - SCL -> SCL2

  - SDA -> SDA2

- Push Button

  - Input Capture -> RP5 (pull-up resistor)

- 5V DC Source (INSTEK GW3030D)

  - 5V DC -> VM

- Motor + Fan

  - A01

  - A02

Full Documentation

finalproject_main.c

void pic24_init() - Purpose: Sets the clock speed of PIC24 to 16 MHz, and all pins to digital. - Arguments: None. - Outputs: None. int main(void) - Purpose: Runs all initialization functions except button initialization, displays real-time temperature, and compares the set temperature to decide whether to initialize the button to control the motor speed. - Arguments: None. - Outputs: Main function of the program and no integer outputs.

TempLib.c

void tempInit(void) - Purpose: Initializes SHT40, sets I2C rate to 100KHz, and writes temp data into sensor by high precision. - Arguments: None. - Outputs: None. void tempRead(unsigned char data[], int dataSize) - Purpose: Master receives data[] from sensor by reading temp data. - Arguments: An array data[] to save temperature data bytes, and dataSize determines the array’s length. - Outputs: None.

MotorLib.c

void initMotor(void) - Purpose: Initialization of the TB6612FNG Motor Driver and sets OC1RS to 0. - Arguments: None. - Outputs: None. void initPushButton(void) - Purpose: Initializes button to detect falling edge by pressing using Input Capture. - Arguments: None. - Outputs: None. void setSpeed(int speed) - Purpose: Changes value of speedMode and motor speed modes by changing OC1RS of PWM in Output Compare. - Arguments: The speed Mode to be changed. - Outputs: None. void _IC1Interrupt(void) - Purpose: Changes the speed mode as the button was pressed and resets the flag to 0. - Arguments: None. - Outputs: None.

final_LCDLib.c

void delay_ms(unsigned int ms) - Purpose: Used for delay in millisecond. - Arguments: unsigned integer in millisecond. - Outputs: None. void lcd_cmd(char Package) - Purpose: Sends an instruction byte to the LCD to configure it according to the peripheral data sheet. - Arguments: A package in char type. - Outputs: None. void lcd_init(void) - Purpose: Initializes the LCD. - Arguments: None. - Outputs: None. void lcd_setCursor(char x, char y) - Purpose: Sets the cursor from which characters can be set on the LCD. - Arguments: Column x and Row y. - Outputs: None. void lcd_printStr(const char *s) - Purpose: Prints a series of characters on the LCD based on the cursor. - Arguments: The string in char type to be printed. - Outputs: None.

The software block diagram is shown below in Fig.2,  which shows how the different functions interact with each other.

Figure 2: Software Block Diagram of the Program

Basic Usage Example:

For the basic usage test, the room temperature is displayed on the LCD to test its functionality. The temperature is changed by pressing a fingertip to the sensor, and the LCD display is checked to view the change in temperature data. Using OC1RS in Output Compare, the motor driver is instructed to run the DC motor at a constant speed. This procedure ensures that the individual devices are working as intended.

Advanced Usage Example:

In the advanced usage test, a button is added to the system and Input Capture is set to trigger on falling edges. The IC Interrupt is called every time the button is pressed, where the speed mode of the motor is changed. The button is then tested to see whether the speed of the fan changes based on presses. The next step is to implement logic in the main function so that the fan only turns on when it goes above a certain temperature. The button press is disabled and the fan is turned off when it falls below the threshold. The value is set to 30 degrees Celsius. A hairdryer is used to raise the temperature, and the DC motor is checked to see whether it starts spinning automatically. The LCD display is checked to see if the temperature values still update when the fan is running. It is made sure that the fan turns off and the button press does not work when the temperature drops below 30. This procedure verifies the operation of the different components when working together.

Header Files

final_TempHeader.h

#ifndef FINAL_TEMPHEADER_H #define FINAL_TEMPHEADER_H #ifdef __cplusplus extern "C" { #endif void tempInit(void); void tempRead(unsigned char data[], int dataSize); #ifdef __cplusplus } #endif #endif /* FINAL_TEMPHEADER_H */

motorHeader.h

#ifndef MOTORHEADER_H #define MOTORHEADER_H #ifdef __cplusplus extern "C" { #endif void setSpeed(int speed); void initMotor(void); void initPushButton(void); void __attribute__((interrupt, auto_psv)) _IC1Interrupt(void); #ifdef __cplusplus } #endif #endif /* MOTORHEADER_H */

final_LCDHeader.h

#ifndef FINAL_LCDHEADER_H #define FINAL_LCDHEADER_H #ifdef __cplusplus extern "C" { #endif void delay_ms(unsigned int ms); void lcd_cmd(char Package); void lcd_init(); void lcd_setCursor(char x, char y); void lcd_printChar(char Package); void lcd_printStr(const char *s); #ifdef __cplusplus } #endif #endif /* FINAL_LCDHEADER_H */

C Libraries

TempLib.c

#include <xc.h> #include <stdio.h> #include "final_LCDHeader.h" void tempInit(void){ // temp write I2C2CONbits.I2CEN = 0; // disable i2c I2C2BRG = 0x9D; // IC2 speed/rate: 100KHz from the Data sheet I2C2CONbits.I2CEN = 1; // enable i2c IFS3bits.MI2C2IF = 0; // clear interrupt flag I2C2CONbits.SEN = 1; // start sending while(I2C2CONbits.SEN == 1); IFS3bits.MI2C2IF = 0; I2C2TRN = 0b10001000 ; // slave address 0x44 and w while(I2C2CONbits.SEN == 1); IFS3bits.MI2C2IF = 0; I2C2TRN = 0b11111101 ; // write command 0xfd (measure T & RH with high precision) while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2CONbits.PEN = 1; // send STOP while(I2C2CONbits.PEN == 1); // wait for STOP to finish } void tempRead(unsigned char data[], int dataSize){ // // temp read IFS3bits.MI2C2IF = 0; // Clear interrupt flag // send START I2C2CONbits.SEN = 1; // send START while(I2C2CONbits.SEN == 1); // wait for START to finish IFS3bits.MI2C2IF = 0; // clear interrupt flag // send slave address with W bit I2C2TRN = 0b10001000 ; // slave address 0x44. write command while(!IFS3bits.MI2C2IF); // wait for ACK IFS3bits.MI2C2IF = 0; // clear interrupt // send read register I2C2TRN = 0xfd; // write command 0xfd (measure T & RH with high precision) while(!IFS3bits.MI2C2IF); // wait for ACK IFS3bits.MI2C2IF = 0; // clear interrupt delay_ms(10); // send resend bit I2C2CONbits.RSEN = 1; // repeated start bit while(I2C2CONbits.RSEN == 1); // wait for the resend bit to be 0 IFS3bits.MI2C2IF = 0; // clear interrupt // send slave address with R bit I2C2TRN = 0b10001001; // slave address 0x44. read command while(!IFS3bits.MI2C2IF); // wait for ACK IFS3bits.MI2C2IF = 0; // clear interrupt // get data I2C2CONbits.RCEN = 1; // start receiving int i; delay_ms(10); for (i = 0; i < 5; i++) { data[i] = I2C2RCV; // save temp bytes in data[] while(!IFS3bits.MI2C2IF); } // send NACK I2C2CONbits.ACKDT = 1; // use NACK I2C2CONbits.ACKEN = 1; // start NACK sequence while(I2C2CONbits.ACKEN == 1); // wait for NACK to finish sending // send STOP I2C2CONbits.PEN = 1; // send STOP while(I2C2CONbits.PEN == 1); // wait for STOP to finish }

MotorLib.c

#include <xc.h> #include "final_LCDHeader.h" int speedMode = 0; void setSpeed(int speed){ // change the speed of fan when fan turns on if (speed==0){ // mode: 0, stop speedMode = 0; OC1RS = 0; } else if (speed==1){ // mode: 1, low speed (also is the initialization speed) speedMode = 1; OC1RS = 12000; } else if (speed==2){ // mode: 2, medium speed speedMode = 2; OC1RS = 24000; } else if (speed==3){ // mode: 3, high speed speedMode = 3; OC1RS = 36000; } }; void initMotor(void){ // in1(RB8):H in2(RB7):L PWM(RB6):H STBY(RB9):H -----> clockwise // *****speed control TRISB = 0b1111110000111111; // RB6 through RB9 are set as outputs to motor driver LATB = 0b0000001101000000; // set RB6,8,9 high //PPS __builtin_write_OSCCONL(OSCCON & 0xbf); // unlock PPS RPOR3bits.RP6R = 18; // use Pin RP6 for Output Compare 1 = "18" (Table 10-3) __builtin_write_OSCCONL(OSCCON ¦ 0x40); // lock PPS //TMR3 T3CON = 0; // stop Timer T3CONbits.TCKPS = 1; // PRE 1:8 TMR3 = 0; // Initialize to zero PR3 = 39999; // period for 20 ms with 1:8 prescaler _T3IF = 0; // clear Timer 3 interrupt flag //OC1 OC1CON = 0; // turn off OC1 for now OC1R = 200; OC1RS = 0; // stop at initial (speed mode: 0) OC1CONbits.OCTSEL = 1; // use Timer 3 for compare source OC1CONbits.OCM = 0b110; // output compare PWM w/o faults T3CONbits.TON = 1; // turn on the timer3 now! } void __attribute__((interrupt, auto_psv)) _IC1Interrupt(void){ // every time the push button is pressed, // the speed mode will increase by 1 delay_ms(10); // delay for button debounce _IC1IF = 0; // clear input capture Interrupt if (speedMode==0) setSpeed (1); else if (speedMode==1) setSpeed (2); else if (speedMode==2) setSpeed (3); else if (speedMode==3) // Reset the mode to 0 to stop the fan setSpeed (0); } void initPushButton(void){ // connect push button to RB5 //AD1PCFG & TRIS bits TRISBbits.TRISB5 = 1; // RB5 to input CNPU2bits.CN27PUE = 1; // pull-up resistor //TMR2 T2CON = 0; // stop Timer T2CONbits.TCKPS = 0b11; // PRE 1:256 TMR2 = 0; // initialize to zero PR2 = 62499; // period for 1 sec with 1:256 prescaler _T2IF = 0; // clear timer 2 interrupt flag _T2IE = 0; // disable Timer 2 interrupt //PPS __builtin_write_OSCCONL(OSCCON & 0xbf); // unlock PPS RPINR7bits.IC1R = 5; // Use Pin RP8 = "5", for Input Capture 1 (Table 10-2) __builtin_write_OSCCONL(OSCCON ¦ 0x40); // lock PPS // IC1 IC1CON = 0; // reset IC1 IC1CONbits.ICTMR = 1; // timer 2 IC1CONbits.ICM = 0b010; // capture on every falling edge _IC1IF = 0; _IC1IE = 1; // enable IC1 interrupt T2CONbits.TON = 1; // turn on the timer2 now! }

final_LCDLib.c

#include "xc.h" #include "final_LCDHeader.h" //#define CONTRAST 0b0000 // Make sure the contrast is adjustable void delay_ms(unsigned int ms){ // set X ms delay while(ms-- > 0){ asm("repeat #15998"); asm("nop"); } } void lcd_cmd(char Package){ // Sends an instruction byte to the LCD to configure //it according to the peripheral?s data sheet. //Send START IFS3bits.MI2C2IF = 0; //Clear interrupt flag I2C2CONbits.SEN = 1; //Begin (S)tart sequency while(I2C2CONbits.SEN == 1);// SEN will clear when Start Bit is complete IFS3bits.MI2C2IF = 0; I2C2TRN = 0b01111100; // 8-bits consisting of the slave address and the R/nW bit while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2TRN = 0b00000000; // 8-bits consisting of control byte while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2TRN = Package; // 8-bits consisting of the data byte while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2CONbits.PEN = 1; //Stop bit while(I2C2CONbits.PEN == 1);// PEN will clear when Stop bit is complete } void lcd_init(void){ //Initializes the LCD using the lcd_cmd function by data sheet. I2C2CONbits.I2CEN = 0; //Disable I2C for safe I2C2BRG = 0x9D; // IC2 speed/rate: 100KHz from the Data sheet IFS3bits.MI2C2IF = 0; //Clear interrupt flag I2C2CONbits.I2CEN = 1; //Enable I2C delay_ms(40); lcd_cmd(0b00111000); // function set, normal instruction mode lcd_cmd(0b00111001); // function set, advanced instruction mode lcd_cmd(0b00010100); // interval osc lcd_cmd(0b01010110); // contrast low lcd_cmd(0b01011110); lcd_cmd(0b01101100); // follower control delay_ms(200); lcd_cmd(0b00111000); // function set, normal instruction mode lcd_cmd(0b00001100); // Display On lcd_cmd(0b00000001); // Clear Display delay_ms(1); } void lcd_setCursor(char x, char y){ // Uses the lcd_cmd function to set the cursor from which // characters can be set on the LCD. Uses values x and y, column and // row char cursor = ( (0x40 * y) + x) ¦ 0b10000000; // Sets cursor using x, y lcd_cmd(cursor); }; void lcd_printChar(char Package){ // A test function that sets a single character // on the LCD based on the position of the cursor I2C2CONbits.SEN = 1;// Initiate Start condition while(I2C2CONbits.SEN == 1);// SEN will clear when Start Bit is complete IFS3bits.MI2C2IF = 0; I2C2TRN = 0b01111100; // 8-bits consisting of the slave address and the R/nW bit while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2TRN = 0b01000000; // 8-bits consisting of control byte w/ RS=1 while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2TRN = Package; // 8-bits consisting of the data byte while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2CONbits.PEN = 1; while(I2C2CONbits.PEN == 1);// PEN will clear when Stop bit is complete }; void lcd_printStr(const char *s){ I2C2CONbits.SEN = 1;//START BIT while(I2C2CONbits.SEN == 1);// SEN will clear when Start Bit is complete IFS3bits.MI2C2IF = 0; I2C2TRN = 0b01111100; // 8-bits consisting of the slave address and the R/nW bit while(*s){ if(*(s+1)!='\0'){ // Not the last char while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2TRN = 0b11000000; // 8-bits consisting of control byte w/ RS=1, C0=1 while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2TRN = *(s); // 8-bits consisting of the data byte }else{ //the last char while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2TRN = 0b01000000; // 8-bits consisting of control byte w/ RS=1, C0=0 while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2TRN = *(s); // 8-bits consisting of the data byte } s++; } while(IFS3bits.MI2C2IF == 0); IFS3bits.MI2C2IF = 0; I2C2CONbits.PEN = 1; // STOP BIT while(I2C2CONbits.PEN == 1);// PEN will clear when Stop bit is complete };

Main Function

finalproject_main.c

#include "xc.h" #include <stdio.h> // for sprintf #include "final_LCDHeader.h" #include "final_TempHeader.h" #include "motorHeader.h" // CW1: FLASH CONFIGURATION WORD 1 (see PIC24 Family Reference Manual 24.1) #pragma config ICS = PGx1 // Comm Channel Select (Emulator EMUC1/EMUD1 pins are shared // with PGC1/PGD1) #pragma config FWDTEN = OFF // Watchdog Timer Enable (Watchdog Timer is disabled) #pragma config GWRP = OFF // General Code Segment Write Protect (Writes to program // memory is allowed) #pragma config GCP = OFF // General Code Segment Code Protect (Code protection is // disabled) #pragma config JTAGEN = OFF // JTAG Port Enable (JTAG port is disabled) // CW2: FLASH CONFIGURATION WORD 2 (see PIC24 Family Reference Manual 24.1) #pragma config I2C1SEL = PRI // I2C1 Pin Location Select (Use default SCL1/SDA1 pins) #pragma config IOL1WAY = OFF // IOLOCK Protection (IOLOCK may be changed via unlocking // seq) #pragma config OSCIOFNC = ON // Primary Oscillator I/O Function (CLKO/RC15 functions as // I/O pin) #pragma config FCKSM = CSECME // Clock Switching and Monitor (Clock switching is enabled, // Fail-Safe Clock Monitor is enabled) #pragma config FNOSC = FRCPLL // Oscillator Select (Fast RC Oscillator with PLL module (FRCPLL)) void pic24_init(){ _RCDIV = 0; //Sets 16MHz frequency for PIC24 AD1PCFG = 0xffff; //Sets pins to digital } int main(void) { pic24_init(); // PIC24 initialization first unsigned char tempbits[6]; // save the temp data to [0] and [1] got from temp // sensor unsigned long int temp; // get the long int "fake" temp by tempbits[0] and // tempbits[1] initMotor(); // initialize the Motor, PWM makes it stop // (speedMode: 0) lcd_init(); // initialize the LCD while(1){ tempInit(); // write temp by sensor delay_ms(10); // delay 10 ms until finish read tempRead(tempbits,6); // read temp data in tempbits array with six bytes // length temp = (unsigned long int)tempbits[0]*256+(unsigned long int)tempbits[1]; // get the long int "fake" temp double decTemp; // "real" temp with decimals decTemp = -45.0+175.0*(temp)/65535; // get the "real" temp with decimals from "fake" // temp by particular math calculation if(decTemp>100){ decTemp = 100.0; // set the upper limit (the highest temp is 100 // degree in Celsius) } if(decTemp<0){ // set the lower limit (the lowest temp is 0 degree // in Celsius) decTemp = 0.0; } lcd_setCursor(0,0); lcd_printStr("Temp:"); // LCD display (first line) lcd_setCursor(0,1); // display decimal temp (second line) char realTemp[8]; sprintf(realTemp, "%5.2f D", decTemp); // ?x.xx D? // 5.2 in the format string ?%5.2f? means 5 place holders // for the whole floating-point number, 2 of which are // for the fractional part. lcd_printStr(realTemp); // prints to the LCD with the decimal temp if(decTemp >= 30.0){ // when temp >= 30 degree, makes the fan turn // automatically in low speed // and we can control speed by push button setSpeed(1); // initializes the speed to mode 1((low)) initPushButton(); // initializes the push button } while(decTemp >= 30.0){ // always running till the temp < 30 degree in order to // automatic fan with speed controls delay_ms(10); // Again! delay 10 ms until finish read tempRead(tempbits,6); // Again! update temp data in tempbits array with six // bytes length temp = (unsigned long int)tempbits[0]*256+(unsigned long int)tempbits[1]; decTemp = -45.0+175.0*(temp)/65535; // get the "real" temp with decimals if(decTemp>100){ decTemp = 100.0; // set the upper limit (the highest temp is 100 // degree in Celsius) } if(decTemp<0){ // set the lower limit (the lowest temp is 0 degree // in Celsius) decTemp = 0.0; } lcd_setCursor(0,0); lcd_printStr("Temp:"); // LCD display (first line) lcd_setCursor(0,1); char realTemp[8]; // display decimal temp (second line) sprintf(realTemp, "%5.2f D", decTemp); // ?x.xx D? // 5.2 in the format string ?%5.2f? means 5 place // holders for the whole // floating-point number, 2 of which are for the // fractional part. lcd_printStr(realTemp); // Again! prints to the LCD with the decimal temp if(decTemp < 30.0){ break; // Once temp < 30 degree, dumps out of while loop } } // when temp < 30 degree, makes motor speed // stop(mode: 0) setSpeed(0); CNPU2bits.CN27PUE = 0; // when temp < 30 degree, disables the push button }; return 0; }

Conclusion

This project makes use of three peripherals: Motor Driver, Humidity & Temperature Sensor, and LCD Display. The device libraries allow for the automatic toggle of a fan with a temperature limit, and the manual control of fan speed with the push of a button. Many important concepts were put into practice through working on this project. Some highlights are sending and receiving packets of data using I2C, using Input Capture to detect the falling edge of a button press, and utilizing Output Compare for PWM control. The project helped build experience working with new devices such as the Motor Driver and Humidity & Temperature Sensor. There were difficulties encountered throughout the course of this work. Some errors with the project are that the buttons are not stable enough, resulting in the speed modes changing unexpectedly at times. Currently, a 10 ms delay is implemented for the purpose of button debouncing, but a more robust system could be put in place in the future. The second issue has to do with reading I2C data from the temperature sensor. According to the datasheet, the sensor sends two bytes of temperature data. However, the second byte is not read correctly. Another issue in the project design is that the sensor can quickly rise in temperature, but is slow to dissipate the heat. So the temperature values take a longer time to cool down than to heat up. Overall, the project was a successful implementation of a temperature-controlled fan. It has good practical significance by presenting itself as a viable and cheap solution to systems that require cooling down or improving airflow.